Practical 3

PART A

(PART A : TO BE REFFERED BY STUDENTS)

PC Network TCP/IP Configuration

A.1—Aim:

The purpose of this session is to gather network information (IP address, MAC

address…etc), learn to use the TCP/IP Packet Internet Groper (ping) command,

and learn to use the Traceroute  & other commands  from the workstation.

A.2--- Prerequisite:

Understanding the basic knowledge of TCP/IP, Mac address and router

A.3--- Outcome:

After successful completion of this experiment students will be able to:

- Get IP address through command prompt

- Trace the router and gateway path

- Execute other commands to examine a network

A.4--- Theory:

Internet Protocol Address: "IP" stands for Internet Protocol. Every machine on a network has a unique identifier. Just as you would address a letter to send in the mail, computers use the unique identifier to send data to specific computers on a network. Most networks today, including all computers on the Internet, use the TCP/IP protocol as the standard for how to communicate on the network. In the TCP/IP protocol, the unique identifier for a computer is called its IP address.

- loopback IP address: The loopback IP address is the address used to access itself. The IPv4 designated 127.0.0.1 as the loopback address with the 255.0.0.0 subnet mask. A loopback interface is also known as a virtual IP, which does not associate with hardware interface.

- TCP/IP is the suite of communications protocols used to connect hosts on the Internet.

- A default gateway is a node (a router) on a TCP/IP network that serves as an access point to another network. A default gateway provides an entry point and an exit point in a network.

The basic commands to examine a computer networks are as under:

- Packet Internet Groper (ping) command is used to provide a basic test of whether a particular host is operating properly and is reachable on the network from the testing host.

- The traceroute (tracert) traces all the routers that a network layer packet has to pass through to get to a destination.

- hostname: finding host/domain name and IP address

- ipconfig (Internet Protocol Configuration) for getting network configuration

- netstat : Network connections, routing tables, interface statistics

- nslookup: query DNS lookup name

- pathping

- route

A.5--- Procedure:

Task:

- Gather TCP/IP configuration information

- Ping the Loopback IP address of your computer

- Ping using names like websites

- Trace the route to any website

- Execute other commands to examine a network

- MAC address/ Mobile and laptop speed

- Observe the output and complete PART B of lab manual

- Save and close the file and name it as EXP3_ your Roll no.

(PART - B)

(TO BE COMPLETED BY STUDENTS)

| Roll. No. : N049 | Name: Tarun Tanmay |
| --- | --- |
| Sem/Year : 5/ third year | Batch:3 |
| Date of Experiment : 22-07-2020 | Date of Submission:22-07-2020 |
| Grade |  |

B.1: Procedure of performed experiment

- Gather TCP/IP configuration information

- Ping the Loopback IP address of your computer

- Ping using names like websites

- Trace the route to any website

- Execute other commands to examine a network

PATH PING:

ipconfig:

Netstat:

Nslookup

Route:

- MAC address/ Mobile and laptop speed

LAPTOP:

MAC address: 80-C5-F2-F64A-69

Speed:

MOBILE:

MAC address: 64:A5:C3:55:99:EA

Speed:

B.2: Observations and Learning’s:

In the above experiment, we learnt various commands that will give us details regarding our network. We found out how a request travels from our devices to the server and back. We also checked the speed of internet of out=r laptops and mobile phones.

B.3:  Conclusion:

An error in connecting to a website can be easily decoded using the commands we learnt in this experiment. We can also find the speed of various networks easily.